NATIONAL OCCUPATIONAL STANDARD

MARBLE-NATURAL STONE PRODUCTION OFFICER

LEVEL 5

REFERENCE CODE /

OFFICIAL GAZETTE DATE -NUMBER/

| Occupation: | MARBLE-NATURAL STONE PRODUCTION OFFICER |
| --- | --- |
| Level: | 5 |
| Reference Code: |  |
| Prepared by: | MARBLE NATURAL STONE AND MACHINERY MANUFACTURERS’ UNION OF TURKEY |
| Verified by: | VQA Mining Sector Committee |
| VQA Board of Directors Approval Date/Issue: | Decision dd. ….. and no. ………… |
| Official Gazette Date/Issue: |  |
| Revision Number: | 00 |

TERMS, SYMBOLS AND ABBREVIATIONS

FLOW CHART: Chart showing the phases of production.

APPLICATION: Applying project sizes and shapes determined in a certain scale to the land

TREATMENT PLANT: Special plant where marble natural stone fractures from cutting operation are separated from cutting water.

BLOCK: Properly shaped mass which has economic value and obtained from marble blocks,

NATURAL STONE: It is the name given to rocks formed naturally in nature which can take any shape with specific hand tools in the decorative way, on which all kinds of surface treatment can be performed, which can be polished and which can be cut and which give corner and edge when cut, which permit taking blocks whatever their style of formation.

DIAMOND WIRE: The wire used for cutting purposes and made of thick disc-shaped beads of industrial diamonds lined sequentially with springs with a diameter of approximately 0.7 cm within steel with a diameter of 1 cm on a steel rope.

ISCO: International Standard Classification of Occupations,

ISG (WHS): Occupational health and safety,

CARD INDEX: Compilation of information on cards in accordance with an organized system,

PERSONAL PROTECTIVE EQUIPMENT (PPE): All kinds of tools, instruments, appliances and devices which are worn, put on or hold by the worker and which protect the worker from one or more hazards arising from the work and effect the health and safety of the worker, and which are designed to suit such purpose,

MARBLE: Marble is the rock formed as a result of the re-crystallization of lime stones combined with Calcium Carbonate or Magnesium Calcium by metamorphosing under the effect of pressure and temperature.

QUARRY: The place where marble blocks are produced,

WASTE: Pieces of marble or natural stone which cannot be utilized as block and rubble at the quarry

WASTE DISPOSAL AREA: Specific area for disposal of waste

RISK: Probability of occurrence of a hazardous event and combination of results.

ROTATION: Taking over different assignments in turns,

SOCKET: parts welded onto places with length of 2-5 cm, thickness of 0.4-1 cm, and width of 1 cm allocated at saw teeth formed by placing diamond particles into steel paste,

DISCONTINUITY: The cracked and broken structures in large marble masses,

DANGER: Potential of damage or injury likely to affect the worker or work place and likely to exist in the workplace or to be caused externally,

TIME SCHEDULE: Schedule in which production plan in terms of day, month, or year,

PRODUCTION UNIT: Area where production machinery is installed.

TABLE OF CONTENTS

1.	INTRODUCTION	6

2.	DEFINITION OF THE OCCUPATION	6

2.1.	Definition of the Occupation	6

2.2.	Place of the Occupation in International Classification System	6

2.3.	Regulations on Health, Safety and Environment	6

2.4.	Other legislation relating to the occupation	7

2.5.	Working Environment and Conditions	7

2.6.	Other Requirements Regarding The Occupation	7

3.	OCCUPATIONAL PROFILE	8

3.1.	Duties, Tasks and Performance Criteria	8

3.2.	Tools, Appliances and Equipment Used	17

3.3.	Knowledge & Skills	17

3.4.	Attitudes and Behaviors	18

4.	TESTING, ASSESSMENT AND CERTIFICATION	19

- INTRODUCTION

National occupational standard titled Marble Natural Stone Production Operator (Level 5) was issued by Turkish Association of Marble, Natural Stone and Machine Producers (TÜMMER) assigned as per the provisions of Vocational Qualifications Authority (VQA) Law No.5544 and “Bylaw on Drawing up National Occupational Standards” and “Regulation on the Establishment, Duties and Operation Procedures and Principles of the Sector Committees of Vocational Qualification Authority” introduced pursuant to the aforementioned Law.

National occupational standard titled Marble Natural Stone Production Operator (Level 5) was assessed upon receiving the opinions of the related institutions and organizations in the sector, and approved by VQA Board of Directors upon examination of VQA Mining Sector Committee.

- INTRODUCTION OF THE OCCUPATION

  - Definition of the Occupation

Marble Natural Stone Production Operator (level 5) is the qualified person who plans the operating plan of the plant, realizes the production, establishes administrative and service departments, and executes occupational development activities.

  - Place of the Occupation in International Classification System

ISCO 08: 7113 Stonemasons, stone cutters and carvers

NACE Rev.2: C-PRODUCTION 23-7

  - Regulations on Health, Safety and Environment

Social Security and General Health Insurance Law No.5510

Regulation on Heavy and Dangerous Works (Official Gazette dd. 16.06.2004 and numbered 25494)

Regulation on Occupational Health and Security (OG dd. 11.01.1974 and no. 14765)

By Law on the Health and Safety Conditions when using Business Equipment (OG dd. 11.02.2004 and no. 25370)

By Law on the Usage of Personal Protective Equipment in the Business Places (OG dd. 11.02.2004 and no. 25370)

Bylaw on Safety and Health Signs (OG dd. 23.12.2003 and no. 25325)

By Law on Heavy and Dangerous Works

Circular on Occupational Training of the Workers to Employ in Heavy and Dangerous Works

Regulation on the General Principles of Waste Management

By Law on the Principles and Procedures of Occupational Health and Safety Trainings of Employees

Regulation on Health and Security Measures for Working with Devices with Screens

Bylaw on the Works of Carrying by Hand

By Law on the Environmental Noise

By Law on Preparation, Completion and Cleaning Works

Occupational Safety and Health Regulation

By Law on the Health and Safety Measures to be Taken in Business Place Buildings and Extensions

Bylaw on Business Place Opening and Licenses

Bylaw on the Control of Solid Wastes

By Law on the Protection of Employees from Dangers Caused by Explosive Environments

Regulation on Fire

Furthermore, it is essential to comply with applicable laws, bylaws, and other legislation regarding occupational health, security, and environment and to carry out relevant risk analysis.

  - Other legislation relating to the occupation

Labor Law No. 4857

Social Security and General Health Insurance Law No.5510

Mining Law No.3217

Environmental Law No. 2872

And it is essential to obey other current legislations, laws, statutory rules and by-laws.

  - Working Environment and Conditions

Marble-Natural Stone Production Supervisor (Level 5) mostly works at  optimum temperature, adequately illuminated, humid, dusty, noisy, requiring occupational health and safety measures, closed and/or open plant environment where there are generally machinery operating with dense water.  S/he works in communication with her/his colleagues and relevant officials during process of work, depending on requirements of the work. S/he works in almost all parts of the year as long as environmental conditions allow working.  Marble natural stone quarry worker is mostly active during the working process and may have to deal with heavy materials time to time.

Occupational diseases caused by the environmental conditions such as lumbar pain, rheumatic diseases, heat stroke, common cold, etc. may be seen. Appropriate personal protective equipment is used during the operations.

There are risks of accident and getting injured during performance of works and these require being taken of measures regarding occupational health and security.

  - Other Requirements Regarding The Occupation

Marble and natural stone Production Operator (level 5) must have “Form of Initial Entrance and Periodical Medical Examination for Workers in Heavy and Dangerous Work" report.

- OCCUPATIONAL PROFILE

  - Duties, Tasks and Performance Criteria

| Duties |  |  |  | Tasks |  |  | Performance Criteria |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code |  | Title |  | Code Title |  |  | Code |  | Description |  |
| A | To carry out the activities related to taking OSH, environmental safety and quality-related precautions (to be continued) |  | A.l |  | To be informed of and to participate in trainings/ meetings of environment and OSH | A.1.1 |  | Identifies training needs of him/herself or field personnel within the frame of work experiences and observations, demands his/her superiors. |  |  |
|  |  |  |  |  |  | A.1.2 |  | Makes recommendations in order to meet training needs in line with identifications, ensures that these trainings are carried out. |  |  |
|  |  |  |  |  |  | A.1.3 |  | Participates in trainings organized according to requirements. |  |  |
|  |  |  |  |  |  | A.1.4 |  | Evaluates trainings from the aspect of qualification. |  |  |
|  |  |  |  |  |  | A.1.5 |  | Gives feedback to the authorities for the evaluations of trainings. |  |  |
|  |  |  | A.2 |  | To ensure that the employees follow OHS and environmental rules, and to take precautions related to risks and hazards | A.2.1 |  | Repeats informing employees periodically by providing information on risks and hazard to arose in work environment. |  |  |
|  |  |  |  |  |  | A.2.2 |  | Attends to the studies conducted to evaluate the risk factors and obtains information and skills for reducing the same. |  |  |
|  |  |  |  |  |  | A.2.3 |  | Ensures the employees receive on-the-job training on the machines and appliances in order to ensure the safety of employees. |  |  |
|  |  |  |  |  |  | A.2.4 |  | Informs on situations requiring emergency action and on actions to be taken, prepares the instructions and ensures that instructions are followed. |  |  |
|  |  |  |  |  |  | A.2.5 |  | Informs on personal protective equipment and its use, prepares the instructions and ensures that the instructions are followed. |  |  |
|  |  |  |  |  |  | A.2.6 |  | Provides information on safe use of equipment and appliances required for work, prepares the instructions and ensures that the instructions are followed. |  |  |
|  |  |  |  |  |  | A.2.7 |  | Ensures that employees learn and follow instructions by informing them on warning signs which are required to be present in work environment, prepares the instructions and ensures that the instruction are learned and followed. |  |  |

| Duties |  |  | Tasks |  |  | Performance Criteria |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Title |  | Code Title |  |  | Code |  | Description |  |
| A |  | To carry out the activities related to taking OSH, environmental safety and quality-related precautions |  | A.2 | To ensure that the employees follow OHS and environmental rules, and to take precautions related to risks and hazards |  | A.2.8 |  | Plans the practices which will enable the personnel to apply the theoretical information in practice on environmental and occupational safety, prepares the instructions, and ensures that the instructions are followed. |
|  |  |  |  |  |  |  | A.2.9 |  | Provides information on use of work environment clearly, orderly and in conformity with hygienic rules by the employees, prepares the instructions and ensures that the instructions are followed. |
|  |  |  |  |  |  |  | A.2.10 |  | Ensures that they are controlled all equipment related to work at working area in compliance with the instructions of occupational health and safety specialist or technical supervisor. |
|  |  |  |  | A.3 | To notify the concerned people of deficiencies on environment and OHS |  | A.3.1 |  | Identifies deficiencies related to OHS and environment such as insufficient lighting, warning sign and protective equipment which may imperil worker's safety at work. |
|  |  |  |  |  |  |  | A.3.2 |  | Notifies concerned people in writing of non resolved deficiencies and ensures that deficiencies are resolved. |
|  |  |  |  |  |  |  | A.3.3 |  | Demands on improvement, change or renewal of personal protective equipment and appliances which are non-suitable and unsafe to use and ensures that necessary actions are taken. |
|  |  |  |  |  |  |  | A.3.4 |  | Enables the provision and use of equipment required for communication in emergency case. |

| Duties |  | Tasks |  | Performance Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Code Title |  | Code | Description |
| B | To plan plant operational organization (To be continued) | B.1 | To plan the deployment on the land and ensure progress and completion of construction (To be continued) | B.1.1 | Determines the area where the plant will be located by observing issues related to raw materials, infrastructure, logistics, employment, sustainability, and market opportunities, in coordination with her/his superiors. Determines and plans production unit, production quantity, development, and enlargement direction. |
|  |  |  |  | B.1.2 | Prepares / gets prepared feasibility report in conjunction with production scale, in coordination with her/his superiors. |
|  |  |  |  | B.1.3 | Determines and applies for the supports and incentives, which can be obtained, according to feasibility report. |
|  |  |  |  | B.1.4 | Carries out the construction by providing the permit, license, material supply for the construction of the closed area. |
|  |  |  |  | B.1.5 | Applies the administrative building to the site as it’s drawn in the layout plan; determines what is required for the construction of the administrative building and carries out the construction. |
|  |  |  |  | B.1.6 | Associates the stockpile area to the production units in accordance with the final production objectives and applies it to the site. |
|  |  |  |  | B.1.7 | Plans the location of block loading and unloading outer cranes by being effective throughout the stockpile area and considering the final stock amount. |
|  |  |  |  | B.1.8 | Plans the infrastructure of the outer cranes by their capacities and carries out the construction. |
|  |  |  |  | B.1.9 | Associates the area required for selection and packaging department as per production amount to the production unit, plans and constructs such area. |
|  |  |  |  | B.1.10 | Determines the capacity and area of the treatment unit and constructs such unit. |
|  |  |  |  | B.1.11 | Determines the location of the building as per the transformer’s capacity and constructs the building. |
|  |  |  |  | B.1.12 | Ensures the installation of a generator with the capacity of supplying the required machinery group and to be engaged instantly in case of power failures. |
|  |  |  |  | B.1.13 | Determines the location and size of the water tank as per the estimated potable water amount and ensures that the water tank is constructed. |

| Duties |  | Tasks |  | Performance Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Code Title |  | Code | Description |
| B | To plan plant operational organization (To be continued) | B.1 | To plan layout in the plant area, to ensure that the construction is carried out. | B.1.14 | Determines the area to dispose the wastes resulting from production and treatment and ensures that such area is used. . |
|  |  |  |  | B.1.15 | Provides the offers for material and labour, determines and reports the optimum offers (providing the optimum conditions) for the company. |
|  |  | B.2 | To purchase machinery and to locate them according to the layout plan | B.2.1 | Makes the primary machinery list as per the time schedule. |
|  |  |  |  | B.2.2 | Determines the domestic and foreign manufacturers of the machinery included in the list and provides their proforma invoices. |
|  |  |  |  | B.2.3 | Informs the information he/she obtains to his/her superiors in written. |
|  |  |  |  | B.2.4 | Ensures the purchase of the machinery according to the instructions of his/her superiors. |
|  |  |  |  | B.2.5 | Prepares the machinery layout and production flow diagram in the production unit. |
|  |  |  |  | B.2.6 | Ensures that the layout infrastructure is prepared in accordance with the criteria determined for the machinery and the machinery is located. |
|  |  |  |  | B.2.7 | Ensures that the machinery’s electricity, water, shore connections are made and testing studies are carried out. |
|  |  |  |  | B.2.8 | Ensures that the machinery’s any kind of calibrations are carried out and machinery is made ready for production. |
|  |  |  |  | B.2.9 | Determines and reports to those concerned the minimum amount of spare parts and consumables required to be kept in stock. |
|  |  |  |  | B.2.10 | Creates a spare part stock system according to the machinery defect statistics and ensures that the system operates problem-free. |
|  |  |  |  | B.2.11 | Ensures that periodic machinery maintenance and repair card indexes are prepared and followed up. |

| Duties |  | Tasks |  | Performance Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Code Title |  | Code | Description |
| B | To plan plant operational organization | B.3 | To determine and organize working teams | B.3.1 | Makes the planning of employee and auxiliary staff in line with the production objectives. |
|  |  |  |  | B.3.2 | Informs concerned people in writing of the provision of identified staff requirements and ensures that it is provided. |
|  |  |  |  | B.3.3 | Measures and determines the abilities and experiences of the employees by making interview and having them practice according to machine and appliances to be used by employees and staff; ensures that they receive the certificates (heavy construction equipment, forklift, cook, crane operator, heater operator and etc.) required by law and/or participate in the periodic trainings. |
|  |  |  |  | B.3.4 | Forms personal information file of the staff as a result of such measurements. Records results and determination of the interviews and practices in this file. |
|  |  |  |  | B.3.5 | Prepares assignment instructions and determines the plant surface treater and plant cutters. |
|  |  |  |  | B.3.6 | Ensures that the works are started in accordance with his/her instructions. |
|  |  |  |  | B.3.7 | Makes the planning of rotation among teams and employees during the work. |
|  |  |  |  | B.3.8 | Makes the planning of the leaves of the employees. |
|  |  |  |  | B.3.9 | Ensures and approves that the scoring and leaves of the employees are recorded. |
|  |  |  |  | B.3.10 | Ensures that the employees receive technical information and quality trainings regarding orientation and machinery in the beginning and periodically. |
|  |  |  |  | B.3.11 | Determines and nominates the required and obligatory consultancies in accordance with the legislation. |
|  |  | B.4 | To share information with the public and civilian authorities | B.4.1 | Submits the subject of work, layout area, number of employees and any kind of identities, documents and information to the associated civilian authorities and security and police forces. |
|  |  |  |  | B.4.2 | Submits the information, documents and papers to the associated administrative authorities. |
|  |  |  |  | B.4.3 | Regularly informs the relevant authorities regarding the work. |

| Duties |  |  | Tasks |  |  | Performance Criteria |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Title |  | Code Title |  |  | Code |  | Description |  |
| C |  | To make production (To be continued) |  | C.1 | To supply marble, natural stone, blocks and rubble stones. |  | C.1.1 |  | Ensures that the amount of blocks and rubble stones required to be made available in the stockpile area in accordance with the daily, weekly and monthly production objectives. |
|  |  |  |  |  |  |  | C.1.2 |  | Ensures that it’s asked about the price in purchasing blocks and rubble stones in accordance with selection, amount, measurement and productivity and the blocks fulfilling the optimum conditions for the company are purchased. |
|  |  |  |  |  |  |  | C.1.3 |  | Prepares the instructions for locating blocks and rubble stones for cut-rock and ST in the stockpile area and ensures that such instructions are followed. |
|  |  |  |  | C.2 | To make production as per the customer request form |  | C.2.1 |  | Reviews the available customer request forms. |
|  |  |  |  |  |  |  | C.2.2 |  | Compares the production schedule with the amount and delivery periods stated in the customer request forms; approves unless there’s failure and ensures that the schedule is completed. |
|  |  |  |  |  |  |  | C.2.3 |  | Determines the third occupant samples and ensures that it’s mutually signed. |
|  |  |  |  |  |  |  | C.2.4 |  | Informs and obtains the approvals of the requesters in case of failure. |
|  |  |  |  |  |  |  | C.2.5 |  | Explains the situation and explains and assures the ways of performance of the work under the optimum conditions. |
|  |  |  |  |  |  |  | C.2.6 |  | Controls and ensures the supply of adequate blocks and rubbles stones in terms of amount, type of marble and plate sizes in the orders. |
|  |  |  |  |  |  |  | C.2.7 |  | Ensures that plate, dimensional marble or tile lines are activated as per the types of order. |
|  |  |  |  | C.3 | To determine and provide the factors to increase productivity (To be continued) |  | C.3.1 |  | Ensures that all the manufacturing machines operate with maximum performance by providing that the speeds of all the manufacturing machines are continuously followed up. |
|  |  |  |  |  |  |  | C.3.2 |  | Ensures that the productivity of the machines are followed up in terms of type and hardness of stones; records the results and informs those concerned. |
|  |  |  |  |  |  |  | C.3.3 |  | Investigates the reasons for failures observed in the production according to the determined objectives and ensures that such failures are recovered. |
|  |  |  |  |  |  |  | C.3.4 |  | Ensures that the socket and diamond paste ratios are determined to maximize the cutting in types of marble and natural stones by carrying out tests. |

| Duties |  |  | Tasks |  |  | Performance Criteria |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Title |  | Code Title |  |  | Code |  | Description |  |
| C |  | To make production |  | C.3 | To determine and provide the factors to increase productivity. |  | C.3.5 |  | Determines the maximum productivity period of consumables and ensures that the consumables are replaced within such periods. |
|  |  |  |  |  |  |  | C.3.6 |  | Organizes the R&D studies on the new product types, machine production rate and consumables. |
|  |  |  |  | C.4 | To select and pack materials |  | C.4.1 |  | Assesses the color, pattern, texture in the marbles and natural stones produced and prepares the selection instruction. |
|  |  |  |  |  |  |  | C.4.2 |  | Selects stone samples by the number of selections and ensures that such stones are placed in the sample frames on the selection table. |
|  |  |  |  |  |  |  | C.4.3 |  | Ensures that the marbles and natural stones selected based on the selection samples are placed in the related cases by their classes. |
|  |  |  |  |  |  |  | C.4.4 |  | Ensures that the cases are transported to the packaging unit and placed in their specified locations by forklifts. |
|  |  |  |  |  |  |  | C.4.5 |  | Ensures that the cases left open in the packaging unit are delivered to the orderers by using third occupant samples in accordance with the delivery instructions. |
|  |  |  |  |  |  |  | C.4.6 |  | Ensures that the required technical files and marks related to the circulation of the products delivered are attached on the package cases. |
|  |  |  |  | C.5 | To deliver product |  | C.5.1 |  | Ensures that the number of delivered cases are added to the sum total and recorded to the bill of lading with their case numbers. |
|  |  |  |  |  |  |  | C.5.2 |  | Ensures that the completed orders are dispatched with the related copy of bill of lading. |
|  |  |  |  |  |  |  | C.5.3 |  | Informs the accounting department regarding the delivery. |

| Duties |  |  |  | Tasks |  |  | Performance Criteria |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code |  | Title |  | Code Title |  |  | Code |  | Description |
| D | To establish administrative and service departments |  | D.1 |  | To establish pre-accounting department. | D.1.1 |  | Measures all expenses, makes cost calculation. |  |
|  |  |  |  |  |  | D.1.2 |  | Determines number of staff required for pre-accounting, makes job announcement, recruits. |  |
|  |  |  |  |  |  | D.1.3 |  | Prepares the task instructions of pre-accounting, informs his/her staff. |  |
|  |  |  |  |  |  | D.1.4 |  | Ensures that the staff works as instructed and audits them periodically. |  |
|  |  |  |  |  |  | D.1.5 |  | Transfers all documents, signed, related to expenses s/he made as of s/he took up the position to pre-accounting, makes them recorded. |  |
|  |  |  |  |  |  | D.1.6 |  | Puts a steel safe which has sufficient weight for preservation of cash and valuable documents at the accounting department. |  |
|  |  |  |  |  |  | D.1.7 |  | Ensures that all employees, machine and appliances, building areas, all material and tools at the building area are insured by receiving tenders against fire, flood, earthquake, occupational accidents and all similar hazards. |  |
|  |  |  | D.2 |  | To establish marketing, sales and purchase departments. | D.2.1 |  | Determines number of staff required, makes job announcement, recruits. |  |
|  |  |  |  |  |  | D.2.2 |  | Evaluates marble and natural stone knowledge of the marketing, sales and purchase staff, gives training and information to make up their deficiencies. |  |
|  |  |  |  |  |  | D.2.3 |  | Transfers all documents related to marketing, sales and purchase which s/he has performed since s/he took up the position against signature with instructions to the employee whom s/he charged. |  |
|  |  |  | D.3 |  | To establish human resources department. | D.3.1 |  | Determines number of staff required, makes job announcement, recruits. |  |
|  |  |  |  |  |  | D.3.2 |  | Gives and audits the instruction for new or present employees' personal rights, conducting labor productivity and performance measurement of the employees. |  |

| Duties |  |  | Tasks |  |  | Performance Criteria |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code | Title |  | Code Title |  |  | Code |  | Description |  |
| E |  | To carry out activities regarding occupational development and their management |  | E.1 | To follow up the occupational publications. |  | E.1.1 |  | Selects, follows up the professional chambers and other relevant domestic/ foreign technical publications and ensures that the determined staff follows up such publications. |
|  |  |  |  |  |  |  | E.1.2 |  | Provides and watches the practices and the visual material about production methods and ensures they are also watched by others (DVD, VCD etc.). |
|  |  |  |  |  |  |  | E.1.3 |  | Follows up and participates in the professional seminars, conferences, congresses and symposiums. |
|  |  |  |  | E.2 | To follow up the occupational innovations for carrying out R&D studies. |  | E.2.1 |  | Follows up and visits professional fairs. |
|  |  |  |  |  |  |  | E.2.2 |  | Participates in occupational development and integration courses and trainings, ensures that the information obtained are disseminated within the working environment. |
|  |  |  |  |  |  |  | E.2.3 |  | Becomes a member of the relevant professional chamber and associations. |
|  |  |  |  |  |  |  | E.2.4 |  | Uses the internet efficiently in order to receive information for occupational development and carry out R&D studies. |
|  |  |  |  |  |  |  | E.2.5 |  | Follows up the innovations in machinery, tools & equipment and consumables, follows the ones that can prove useful in the working environment. |
|  |  |  |  | E.3 | To follow up the laws, regulations and legislations on the occupation. |  | E.3.1 |  | Learns about the legislations of mining in various fields such as mining, environment, forestry, agriculture. |
|  |  |  |  |  |  |  | E.3.2 |  | Determines necessary permits for work in accordance with the related legislation's provisions and ensures that they are obtained. |

  - Tools, Appliances and Equipment Used

- Communication tools (radio set, telephone etc.)

- Scale

- Personal protective equipment (helmet, boot, earplug, glasses, dust glasses, dust mask, work shoes, gloves, etc.)

- Basic hand tools (voltage tester, screwdriver, hammer and wrench set etc.)

- Warning signs and plates

  - Knowledge & Skills

- Knowledge of emergency

- Knowledge on tools, apparatus and equipment

- Basic first aid knowledge

- Knowledge on computer usage

- Knowledge on land use

- Knowledge and skill of drawing

- Ability to manage team

- Knowledge and skill of using hand tools

- Manual skill

- Visual skill

- Knowledge of maps

- Ability of persuasion

- Communication skill

- Knowledge on human psychology

- Knowledge on Occupational Health and Safety Precautions

- Knowledge on the work site operation procedures

- Knowledge on quality control principles

- Ability of decision making

- Knowledge on use of user's guide, handbook and maintenance book

- Knowledge on materials

- Ability of distance and weight estimation

- Knowledge on legal regulations regarding the occupation

- Knowledge on vocational electricity

- Knowledge on vocational physics

- Knowledge on vocational geology

- Knowledge on vocational mathematics

- Knowledge on occupational and technological advancements

- Knowledge of vocational terms

- Ability of note taking

- Ability of working in a team or an organization

- Skill of learning and being able to share what s/he learned

- Ability to learn and teach

- Knowledge on measuring and control

- Ability of planning and problem solving

- Knowledge on colors

- Knowledge on standard materials

- Knowledge and skill of technical drawing

- Knowledge on warning sign and table

- Knowledge on product

- Knowledge on fire fighting techniques and use of fire-extinguisher

- Knowledge on spare parts and consumables

- Ability to find directions

- Knowledge of surface

  - Attitudes and Behaviors

- To be cold blooded and calm under emergency and stressful situations

- To pay attention to use of appliances and consumables

- To be researcher

- To be hardworking

- To be able to use working time effectively and efficiently in compliance with work orders and instructions

- To adopt environmental, quality and OHS rules

- To be sensitive to protection of environment

- To be open to change and to adapt to changing conditions

- To pay attention to details

- To be attentive

- To be honest

- Ability to manage team.

- To be open to criticism

- To be energetic

- To be observer

- To be cheerful

- To take initiatives

- To take care of human relations

- To have working discipline

- To pay attention to work safety

- To obey the working principles of the workplace

- To pay attention to cleaning, tidiness and workplace organization

- To pay attention to the usage of tools, appliances and equipments belonging to workplace

- To pay attention to quality

- To be decisive

- To be willingful to improve himself/herself

- To ensure his/her own safety and safety of other people

- To have professional ethics

- Being planned and organized

- To be sensitive to issues of risk and hazard factors

- To be patient

- To take care of his/her own health

- Knowing his/her responsibilities and fulfilling the same

- To learn instructions and guides and to comply them meticulously

- To give fast and right reaction in dangerous situation in his/her own movement area

- To be clean

- To inform superiors properly and in time

- To share information clearly, effectively at shift change

- To be open to innovations

- To inform concerned people about the malfunctions which are not under his/her authority

- To use the time well.

- TESTING, ASSESSMENT AND CERTIFICATION

Testing and assessment for certification with respect to national qualifications based on Marble- Natural Stone Production Supervisor (Level 5) Occupational Standard shall be held in written and/or oral forms, theoretically and practically, in testing and assessment centers where required conditions are met.

Testing and assessment method and practice principles shall be detailed with national qualifications to be drawn up pursuant to this occupational standard. Activities regarding testing, assessment and certification shall be conducted within the framework of Vocational Qualification Authority, Testing and Certification Regulation.

Occupational Qualification Level is determined as Level Four (4) in the octal (8) level matrix.