NATIONAL OCCUPATIONAL STANDARD

Marble - Natural Stone Surface Treatment Worker

LEVEL 4

REFERENCE CODE / …

OFFICIAL JOURNAL DATE-ISSUE/

| Occupation: | Marble - Natural Stone Surface Treatment Worker |
| --- | --- |
| Level: | 4 |
| Reference Code: | ……………………………………. |
| Prepared by: | ………………………………………… |
| Verified by: | VQA ………. Sector Committee |
| VQA Board of Directors Approval Date/Issue: | ………… Date and ………. Decision No. |
| Official Gazette Date / Issue: | … |
| Revision No: | 00 |

TERMS, SYMBOLS AND ABBREVIATIONS

BLOCK: Properly shaped mass which has economic value and obtained from marble blocks.

NATURAL STONE: It is the name given to rocks formed naturally in nature which can take any shape with specific hand tools in the decorative way, on which all kinds of surface treatment can be performed, which can be polished and be cut and which give corner and edge when cut, which permit taking blocks whatever their style of formation.

DIAMOND WIRE: The wire used for cutting purposes and made of thick disc-shaped beads of industrial diamonds lined sequentially with springs with a diameter of approximately 0.7 cm within steel with a diameter of 1 cm on a steel rope.

ISCO: International Standard Classification of Occupations

OHS: Occupational health and safety,

CARD INDEX: Compilation of information on cards in accordance with an organized system,

PERSONAL PROTECTIVE EQUIPMENT (PPE): All kinds of tools, instruments, appliances and devices which are worn, put on or hold by the worker and which protect the worker from one or more hazards arise from the work and effect the health and safety of the worker, and which were designed to suit such purpose,

MARBLE: Marble is the rock formed as a result of the re-crystallisation of lime stones combined with Calcium Carbonate or Magnesium Calcium by metamorphosing under the effect of pressure and temperature.

QUARRY: The place where marble blocks are produced,

WASTE: Pieces of marble or natural stone which cannot be utilized as block and rubble at the quarry

WASTE DISPOSAL AREA: Specific are for disposal of waste

RISK: The composition of the possibility of any dangerous situation and the results of the same,

ROTATION:  Taking over different assignments in turns,

DISCONTINUITY: The cracked and broken structures in large marble masses,

DANGER: Potential of damage or injury likely to affect the worker or work place and likely to exist in the workplace or to be caused externally

APPLICATION: Implementing project measures and forms to land determined with one scale

PRODUCTION UNIT: Area where machines for production are placed.

TREATMENT PLANT: A private plant, in which marble natural stone particles that mixed with the cutting water during the cutting procedure are removed from the water.

TIME SCHEDULE: Plan which indicates day, month and year for a production.

FLOW CHART: Diagram indicating phases of production.

SOCKET: The pieces, which are produced by placing diamond particles in steel dough, and which are welded to the places removed from the edges of saws that are 2-5 cm long,0.4 -1 cm thick, and 1 cm wide,

TABLE OF CONTENTS

1.	INTRODUCTION	6

2.	INTRODUCTION OF THE OCCUPATION	7

2.1.	Place of the Occupation in International Classification System	7

2.2.	Regulations on Health, Safety and Environment	7

2.3.	Other Legislation regarding the Occupation	8

2.4.	Working Environment and Conditions	8

2.5.	Other Requirements regarding the Occupation	8

3.	OCCUPATIONAL PROFILE	9

3.1.	Duties, Tasks and Performance Criteria	9

3.2.	Tools, Appliances and Equipment Used	16

3.3.	Knowledge & Skills	16

3.4.	Attitudes and Behaviors	17

4.TESTING, ASSESSMENT AND CERTIFICATION	18

- INTRODUCTION

National occupational standard titled Marble Natural Stone Sizing and Cutter (Level 4) was issued by Turkish Association of Marble, Natural Stone and Machine Producers (TÜMMER) assigned as per the provisions of Vocational Qualifications Authority (VQA) Law No.5544 and “Bylaw on Drawing up National Occupational Standards” and “Regulation on the Establishment, Duties and Operation Procedures and Principles of the Sector Committees of Vocational Qualification Authority” introduced pursuant to the aforementioned Law.

National occupational standard titled Marble-Natural Stone Quarry Worker (Level 4) was assessed upon receiving the opinions of the related institutions and organizations in the sector, and approved by VQA Board of Directors upon examination of VQA Mining Sector Committee.

- INTRODUCTION OF THE OCCUPATION

Marble-Natural Stone Surface Treatment Worker Level (4) is a qualified person who participates in occupational development activities, manages the section of cutting in production and makes work organization by taking precautions related to occupational health and safety and environment within the frame of quality systems.

  - Place of the Occupation in International Classification System

ISCO 08		: 7113 Stonemasons, stone cutters and carvers

NACE Rev.2	: C-MANUFACTURING  23-7

  - Regulations on Health, Safety and Environment

Social Security and General Health Insurance Law No.5510

Regulation on Heavy and Dangerous Work (Date of Official Gazette: 6/16/04, Issue of Official Gazette: 25494)

Bylaw on Occupational Health and Safety (Date of Official Gazette: 1/11/74, Issue of Official Gazette: 14765)

Regulation on Health and Safety Conditions in Use of Work Equipments (Date of Official Gazette: 2/11/04, Issue of Official Gazette: 25370)

Regulation on Use of Personal Protective Equipments in the Workplace (Date of Official Gazette:2/11/04, Issue of Official Gazette:25370)

Safety and Health Signs Regulation(Date of Official Gazette:23.12.2003, Issue of the Official Gazette:25325)

Regulation on Heavy and Dangerous Works

Notification on the professional Trainings of the Workers to be Employed in Heavy and Dangerous Works

Regulation on the General Principles of Waste Management

Regulation on Procedures and Principles of Occupational Health and Safety Trainings of the Employees

Regulation on Health and Security Measures for Working with Devices with Screens

Regulation on Manual Handling Works

Regulation on Noise

Regulation on Preparation, Completing and Cleaning Works

Occupational Safety and Health Regulation

Regulation on Health and Safety Measures to be taken in the Workplace Buildings and Additional Buildings

Regulation on Business Licenses

Regulation of the Control of Solid Wastes

Regulation on Preventing the Personnel from the Hazards of the Explosive Environments

Regulation on Fire

*Furthermore it is essential to obey legislation, laws, bylaws and regulations on occupational health and safety and environment; and to perform risk analysis regarding this issue.

  - Other Legislation regarding the Occupation

Labor Law No. 4857

Social Security and General Health Insurance Law No.5510

Mining Law No.3217

Environmental Law No. 2872

And it is essential to obey other current legislations, laws, statutory rules and by-laws.

  - Working Environment and Conditions

Marble Natural Stone Surface Treatment Worker (level 4) works at appropriate temperature in open or closed factory environment that includes machines working with a large quantity of water, which is adequately lighted, humid, dusty, and noisy; and where occupational health and safety measures are required to be taken. S/he works in communication with his/her co-workers and relevant authorities, depending on the requirements of work; works almost any time of the year as long as atmospheric conditions are convenient; mostly is on the move in the process of work and sometimes must stand on feet for a long time and carry heavy materials; may face occupational diseases like backache, knee pain, rheumatism diseases etc. due to the environmental conditions. Furthermore, appropriate personal protective equipment is used during the operations. There are also damage and injury risks which require taking occupational health and safety measures while practicing the work.

  - Other Requirements regarding the Occupation

Marble Natural Stone Surface Treatment Worker (level 4) must have the “Form of Initial Entrance and Periodical Medical Examination for Workers in Heavy and Dangerous Works”.

- OCCUPATIONAL PROFILE

  - Duties, Tasks and Performance Criteria

| Duties |  | Tasks |  | Performance Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Code | Title | Code | Description |
| A | To apply and follow Occupational Health & Safety Rules | A.1 | To be informed of and to participate in trainings/ meetings of environment and OSH | A.1.1 | Creates conditions by taking precautions necessary for meeting the training needs of employees. |
|  |  |  |  | A.1.2 | Ensures that the trainings are received. |
|  |  |  |  | A.1.3 | Informs and demands his/her superiors on training subjects s/he identified or needed related to work. |
|  |  |  |  | A.1.4 | Participates in trainings organized according to requirements. |
|  |  |  |  | A.1.5 | Observes how much the field personnel and other employees according to the working hierarchy benefited and determines the deficiencies. |
|  |  |  |  | A.1.6 | Makes recommendations by notifying deficiencies of his/her superiors in written. |
|  |  | A.2 | To ensure that the employees follow OHS and environmental rules, and to take precautions related to risks and hazards | A.2.1 | Ensures that the employees follow instructions by informing them periodically on risks and hazards to arose in work environment. |
|  |  |  |  | A.2.2 | Attends to the studies conducted to evaluate the risk factors and obtains information and skills for reducing the same. |
|  |  |  |  | A.2.3 | By informing employees on hazards likely to imperil their safety and health at work, |
|  |  |  |  | A.2.4 | Ensures that employees follow instructions, by informing them on situations requiring emergency action and on actions to be taken. |
|  |  |  |  | A.2.5 | Ensures that personal protective equipments are used in compliance with instructions, by giving information on these equipment and their use. |
|  |  |  |  | A.2.6 | Ensures that personal protective equipment required for work is delivered to employees with delivery bill. |
|  |  |  |  | A.2.7 | Provides information on safe use of equipment and appliances required for work, ensures that instructions are followed. |
|  |  |  |  | A.2.8 | Ensures that employees learn and follow instructions by informing on warning signs which are required to be present in work environment. |
|  |  |  |  | A.2.9 | Ensures that planned practices, which will enable the personnel to apply the theoretical information on environmental and occupational safety to practice, are performed, and participates in practices. |
|  |  |  |  | A.2.10 | Ensures that employees follow instructions on using work environment clearly, orderly and in conformity with hygienic rules. |
|  |  |  |  | A.2.11 | Controls all equipment related to work at working area, in compliance with the instructions of occupational health and safety specialist or technical supervisor, or ensures that they are controlled. |
|  |  | A.3 | To notify concerned people of deficiencies on environment and OHS | A.3.1 | Identifies deficiencies related to OHS and environment such as insufficient lighting, warning sign and protective equipment which may imperil worker's safety at work. |
|  |  |  |  | A.3.2 | Notifies concerned people in written of non resolved deficiencies. |
|  |  |  |  | A.3.3 | Notifies concerned people in written for improvement, change or renewal of personal protective equipment and appliances which are non-suitable and unsafe with its justifications. |
|  |  |  |  | A.3.4 | Enables the use and preparation of equipment required for communication in emergency case. |

| Duties |  | Tasks |  | Performance Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Code | Title | Code | Description |
| B | To organize the work | B.1 | To enable the construction of factory and all outbuildings. | B.1.1 | Determines entrance and exit routes of working area. |
|  |  |  |  | B.1.2 | Ensures the provision of what is required for the location of factory and outbuildings, design and capacity according to employee number and developments likely to occur. |
|  |  |  |  | B.1.3 | Reports all developments to authorities during the construction of factory and outbuildings. |
|  |  |  |  | B.1.4 | Ensures that the employees and auxiliary staff are placed according to planned positions at factory and outbuildings. |
|  |  |  |  | B.1.5 | Puts instructions in specified sections of factory and outbuildings to ensure that the concerned tools are used in a clean and ordered manner, and in conformity with hygienic rules; follows these instructions by ensuring compliance to them. |
|  |  | B.2 | To provide required machine and appliances. | B.2.1 | Enables the provision of necessary cutting machines, appliances and consumables from company storages if available. |
|  |  |  |  | B.2.2 | Enables the provision of lacking appliances and consumables under suitable conditions upon making a market research. |
|  |  |  |  | B.2.3 | Ensures that the cutting machines and appliances are ready for operation. |
|  |  |  |  | B.2.4 | Prepares periodical maintenance card index in line with the criteria provided by manufacturer for cutting machines and appliances. |
|  |  |  |  | B.2.5 | Ensures that the spare part storage system is formed and works without any problem, taking into consideration the malfunctions preventing the cutting machinery from working. |
|  |  | B.3 | To compose determined working groups. | B.3.1 | Ensures the provision of planned employee and auxiliary staff. |
|  |  |  |  | B.3.2 | Ensures the provision of identified staff requirements by informing concerned people in written. |
|  |  |  |  | B.3.3 | Ensures that the staff works in compliance with the definition included in the personal information file by starting them to work. |
|  |  |  |  | B.3.4 | Notifies his/her superiors of his/her opinions related to rotation between staff and employees. |
|  |  |  |  | B.3.5 | Notifies his/her superiors of employees’ requests of take a leave. |
|  |  |  |  | B.3.6 | Records score card and leave records of employees daily, weekly and monthly, informs accounting department. |
|  |  | B.4 | To share information with official authorities and civilian authorities. | B.4.1 | Provides all necessary information and document related to work, and shares them with his/her superiors. |

| Duties |  | Tasks |  | Performance Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Code | Title | Code | Description |
| C | To manage surface treatment procedures of production (To be continued) | C.1 | To perform surface procedures on strips or sized tables cut | C.1.1 | Examines routine factory program or order forms. |
|  |  |  |  | C.1.2 | Compares the quantities and delivery times stated in the production program and order forms, and defines the methods to ensure the production is performed under best conditions. |
|  |  |  |  | C.1.3 | Ensures that the routine factory programs and/or orders are carried out. |
|  |  |  |  | C.1.4 | Detects the reasons likely to cause failure, and informs his/her superiors. |
|  |  |  |  | C.1.5 | Receives the observation samples by signature. |
|  |  |  |  | C.1.6 | Requests materials, which are sized in the routine factory program and/or orders according to quantity, marble type, and sizes, from the cutting department or his/her superiors. |
|  |  |  |  | C.1.7 | Ensures that the related surface treatment machines are operated according to the routine factory program and/or order types. |
|  |  |  |  | C.1.8 | Gives instructions on the surface treatment procedures to be performed to his/her subordinates. |
|  |  |  |  | C.1.9 | Gives instructions on communicating the materials produced to the selection table after the production procedure, and ensures that these instructions are followed. |
|  |  |  |  | C.1.10 | Prepares daily, weekly and monthly production reports, and informs his/her superiors in writing. |
|  |  | C.2 | To determine and provide factors increasing productivity, and to ensure the same | C.2.1 | Ensures that all surface treatment machines work with maximum performance, by continuously observing their production rate. |
|  |  |  |  | C.2.2 | Informs his/her superiors in writing by recording the results and observing productivity of the machines according to types and hardness of stone. |
|  |  |  |  | C.2.3 | Ensures that the malfunctions in the production are resolved in accordance with the objectives defined. |
|  |  |  |  | C.2.4 | Makes demand from his/her superiors on determining the characteristics of surface treatment materials to ensure that the surface treatments of marble and natural stone are at maximum level. |
|  |  |  |  | C.2.5 | Identifies the maximum productivity duration of consumables, and ensures that they are replaced in this duration. |

| Duties |  | Tasks |  | Performance Criteria |  |
| --- | --- | --- | --- | --- | --- |
| Code | Title | Code | Title | Code | Description |
| D | To be involved in activities regarding vocational development | D.1 | To follow occupational publications. | D.1.1 | Follows the professional chambers and relevant technical publications. |
|  |  |  |  | D.1.2 | Provides and watches the practices and the visual materials about production methods, and ensures that the employees got informed. |
|  |  |  |  | D.1.3 | Follows and tries to participate in the professional seminars, conferences, congresses and symposiums, related to the occupation. |
|  |  | D.2 | To follow the technological innovations about the occupation. | D.2.1 | Follows and visits professional fairs, related to the occupation. |
|  |  |  |  | D.2.2 | Participates in vocational development and integration courses and trainings, ensures that the information obtained are disseminated within the working environment. |
|  |  |  |  | D.2.3 | Becomes a member of the relevant professional chamber and associations. |
|  |  |  |  | D.2.4 | Uses the internet efficiently in order to receive information for occupational development. |
|  |  |  |  | D.2.5 | Follows the innovations in machinery, tools & equipment and consumables, and examines the same. |
|  |  |  |  | D.2.6 | Detects the innovations likely to be profitable in the work site, and informs the related authorities in written. |
|  |  | D.3 | Follows the laws, regulations and legislations concerning the occupation. | D.3.1 | Learns about the legislations such as mining, environment, forestry, agriculture, concerning the occupation. |

  - Tools, Appliances and Equipment Used

- Communication tools (radio set, telephone etc.)

- Personal protective equipment (helmet, boot, earflap, glasses, dust glasses, dust mask, work shoes, protective gloves, coverall etc.)

- Warning signs and tablets

- Basic hand tools (voltage tester, screwdriver, hammer and wrench set etc.)

  - Knowledge & Skills

Knowledge on emergency situation

Knowledge on tools, apparatus and equipment

Knowledge on computer usage

Knowledge on land use

Knowledge and skill of drawing

Team management skill

Knowledge and skill of using hand tools

Manual skill

Visual skill

Knowledge of maps

Ability of persuasion

Communication skills

Basic first aid knowledge

Knowledge on human psychology

Knowledge on Occupational Health and Safety Precautions

Knowledge on the work site operation procedures

Knowledge on quality control principles

Ability of decision making

Knowledge on use of user's guide, handbook and maintenance book

Knowledge on materials

Ability of distance and weight estimation

Knowledge on legal regulations regarding the occupation

Vocational electric knowledge

Vocational physics knowledge

Vocational geology knowledge

Vocational mathematics knowledge

Knowledge on occupational and technological advancements

Knowledge of vocational terms

Ability of note taking

Ability of working in a team or an organization

Skill of learning and being able to share what s/he learned

Ability to learn and teach

Knowledge on measuring and control

Ability of planning and problem solving

Knowledge on colors

Knowledge on standard materials

Knowledge and skill of technical drawing

Knowledge on warning sign and table

Knowledge on product

Knowledge on fire fighting techniques and use of fire-extinguisher

Knowledge on spare parts and consumables

Ability to find directions

Knowledge of surface

  - Attitudes and Behaviors

To be cold blooded and calm under emergency and stressful situations

To pay attention to use of appliances and consumables

To be researcher

To be hardworking

To be able to use working time effectively and efficiently in compliance with work orders and instructions

To be sensitive to protection of environment

To adopt environmental, quality and OHS rules

To be open to change and to adapt to changing conditions

To pay attention to details

To be attentive

To be honest

Team management skill

To be open to criticism

To be energetic

To be observer

To be cheerful

To take initiatives

To take care of human relations

To have working discipline

To pay attention to work safety

To obey the working principles of the workplace

To pay attention to cleaning, tidiness and workplace organization

To pay attention to the usage of tools, appliances and equipments belonging to workplace

To pay attention to quality

To be decisive

To be willing to improve himself/herself

To ensure his/her own safety and safety of other people

To have professional ethics

To be planned and organized

To be sensitive to issues of risk and hazard factors

To be patient

To take care of his/her own health

To know his/her responsibilities and to fulfill the same

To learn instructions and guides and to comply them meticulously

To give fast and right reaction in dangerous situation in his/her own movement area

To be clean

To inform superiors properly and in time

To share information clearly, effectively at shift change

To be open to innovations

To inform concerned people about the malfunctions not under his/her authority

Efficient time management

- 4. TESTING, ASSESSMENT AND CERTIFICATION

Testing and assessment for certification with respect to national qualifications based on Marble Natural Stone Surface Treatment Worker (Level 4) Occupational Standard shall be held in written and/or oral forms, theoretically and practically, in testing and assessment centers where required conditions are met.

Testing and assessment method and practice principles shall be detailed with national qualifications to be drawn up pursuant to this occupational standard. Activities regarding testing, assessment and certification shall be conducted within the framework of Vocational Qualification Authority, Testing and Certification Regulation.

Note: This part shall not be published on the Official Gazette, shall only be included on the web site of VQA.

Annex: Institutions participated in the Occupational Standard Preparation Process

……….